# Day 01 — Basic Routing, ARP & ICMP Analysis

## Objective

The objective of this lab is to build a small routed network and understand how devices communicate across different IP networks.

In this lab, we will:

- Configure IP addresses, subnet masks, and default gateways on two Linux PCs.

- Configure two Gigabit interfaces on the router.

- Connect two LANs through the router.

- Verify connectivity between PC1 and PC2 using ICMP.

- Observe ARP request/reply messages in Wireshark.

- Analyze ICMP Echo Request and Echo Reply packets.

## Topology

The lab uses the smallest network required to demonstrate communication between two different networks.

PC1 ─── SW1 ─── R1 ─── SW2 ─── PC2

### Devices

- 2 × IOSvL2 Switches

- 1 × Router

- 2 × Linux Hosts

### IP Addressing

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | eth0 | 192.168.10.10 | 255.255.255.0 (/24) | 192.168.10.1 |
| R1 | e0/0 | 192.168.10.1 | 255.255.255.0 (/24) | — |
| R1 | e0/1 | 192.168.20.1 | 255.255.255.0 (/24) | — |
| PC2 | eth0 | 192.168.20.10 | 255.255.255.0 (/24) | 192.168.20.1 |

## Steps

### 1. Configure PC1

Assign the IP address to PC1’s eth0 interface:

ip addr add 192.168.10.10/24 dev eth0

Configure the default gateway:

ip route add default via 192.168.10.1

Verify the configuration:

ip addr show eth0
ip route

PC1 is now part of the 192.168.10.0/24 network.

### 2. Configure PC2

Assign the IP address to PC2:

ip addr add 192.168.20.10/24 dev eth0

Configure the default gateway:

ip route add default via 192.168.20.1

Verify the configuration:

ip addr show eth0
ip route

PC2 is now part of the 192.168.20.0/24 network.

### 3. Configure R1 Gi0/0

Configure the router interface connected to SW1:

enable
configure terminal

interface GigabitEthernet0/0
ip address 192.168.10.1 255.255.255.0
no shutdown
exit

This interface acts as the default gateway for PC1.

### 4. Configure R1 Gi0/1

Configure the router interface connected to SW2:

interface GigabitEthernet0/1
ip address 192.168.20.1 255.255.255.0
no shutdown
exit

This interface acts as the default gateway for PC2.

Save the configuration:

end
write memory

### 5. Verify Router Interfaces

Check the status and IP addresses of the router interfaces:

show ip interface brief

The expected result is that both Gi0/0 and Gi0/1 are up/up.

The router should have two directly connected networks:

192.168.10.0/24
192.168.20.0/24

### 6. Test Local Connectivity

From PC1, first test its local gateway:

ping 192.168.10.1

From PC2, test its local gateway:

ping 192.168.20.1

If both pings are successful, the PCs can communicate with their respective router interfaces.

### 7. Test PC1 to PC2 Connectivity

From PC1, ping PC2:

ping 192.168.20.10

The expected result is successful ICMP Echo Replies from:

192.168.20.10

This proves that traffic is successfully traveling:

PC1 → SW1 → R1 → SW2 → PC2

The return traffic follows the reverse path:

PC2 → SW2 → R1 → SW1 → PC1

## Wireshark Analysis

Wireshark is used to observe the actual packets exchanged between the devices.

Capture traffic on both PC–SW links.

### ARP Analysis

Before the first successful ping, ARP is used to discover the MAC address of the next-hop device.

Use the Wireshark display filter:

arp

You should observe an ARP Request followed by an ARP Reply.

The ARP Request contains a message similar to:

Who has 192.168.10.1?

The router responds with an ARP Reply:

192.168.10.1 is-at <MAC address>

The same process occurs on the PC2 side for its gateway:

Who has 192.168.20.1?

This demonstrates that ARP resolves an IP address to a MAC address on the local Ethernet network.

### ICMP Analysis

Use the following Wireshark filter:

icmp

After ARP resolution, ICMP packets can be observed during the ping.

The main ICMP messages are:

Echo Request
Echo Reply

PC1 sends an ICMP Echo Request toward PC2, and PC2 responds with an ICMP Echo Reply.

The Echo Request and Echo Reply can be matched using their:

- Identifier

- Sequence number

This allows us to confirm that the reply corresponds to the original request.

### Key Takeaways

- Devices use ARP to discover MAC addresses on the local network.

- A default gateway is required when a host communicates with another IP network.

- A router forwards packets between different networks.

- ICMP Echo Request and Echo Reply are used by ping to test IP connectivity.